# Product charater:

This single-door controller is a series of independent ones that are developed for the control of exits of office building, intellectual building, residential area, public channel of building, through which user can open door with respondent card, can open with pass code or can with both respondent card and pass code, then a simple door controller system solution program is made. Product character:

working voltage: DC 12V

unlock current: ≤1 A

static current: ≤60MA

available card quantity: 800 pieces

Maximum card-read distance: 5cm-15cm

RF card type: EM or EM compatible card

size: 117×117×21mm

Special attention:

1. please strictly do per user manual.

2. please don’t do with power on! Need pay more special attention

on anode and cathode.

# How to assemble:

Unpack cross-screw that fixes faceplate and motherboard, then take off motherboard and get it assembled on side wall which is at the exit, with attention on vertical orientation.

alert: 1. please ensure to switch off power before connecting lines

and switch on power after all lines being connected well! Please be sure that power voltage is 12v and that anode& cathode are in place!

2. the switch to theftproof is at the left-bottom corner. Buzzer

will keep short-ring alert when back cover is open or spring is loose, and alert will stop once back cover is closed, which is default as close when ex-work, so user can set up on per function.

circuit diagram:

note:

- 1st pin connects power anode+12V and 2nd pin connects cathode, attention: Anode and cathode must not be wrongly connected, or controller is burnt.

- 3rd pin, Push, is signal line and connects Push end of Power,or connects GND of lock.4th pin, LOCK+, is output of +12V voltage,or connects +12VDC of lock.

- 5th and 6th pins connect switch, and 7th and 8th pins contact

door bell without different between anode and cathode.

- to keep door controller work steadily, please don’t connect

electric lock directly with controller.

5.  when J2 jump in 2-3 feet [that performance] for the machine reading card, when J2 jump in 1-2 [Double], can realize the machine and fission coil. Reader1 connect two-way reading card without lights, Reader2 connect the fission coil with lights. J2 fission coil factory default Settings for 2-3 feet [performance] state.

6.  in jumping on 1-2 J1 [NO], shall &gnd disconnect,for the impending state, shall not output any signal, when jumping on J1

2-3 [NC], conduction, shall &gnd GND shall output Signal factory default Settings. J1 for 1-2 feet [NO] state.

programming setup way:

original management pass word is 1688 and user pass word is 8888.

1.enter programming

push*=>input management pass word=>push#

2 function setup(under programming circumstance)

- modify management password:

push 0=>input new management password =>push#=>

againinput new management password =>push#

note: password is 1-10 digital number.

(2) add user card

push 1 =>read card =>continually read car=>push#;

input 10 card serial number =>push#;

use management authority card:read authority card=>read user card => read authority card.

Note:10 card serial number is EM card front 10 10 disables ABA yards

(3) 4 ways to delete user card

A. Delete all user card: push2=>push0000=>push #

B. Delete the card read: push2=>read card=>push#

C. Delete lost card: push2=>10 card serial number =>push#

D. Use a management card delete using: read delete card=> read user card => read delete card

Note: please careful operation "delete all using", delete all using this opportunity to continue when stolid 20 seconds warning.

(4) setup for open mode

A. Open with reading valid card: push 300=>push#

B..Open with reading valid card and password: push 301=>

push#

C. open with reading valid card or entrying correct password:

push302=>push#

note: original setup is 302 for reading valid card or input password to open door.

(5) setup for open door time

push4=>X X=>push#

note: XX is during 01-99, measure unit is second and origin is 3

seconds.

(6) how to modify use password

push 5=>input 1 new password with 4 digital number=>push#

=>again input 1 new password with 4 digital number=>push#

to save

(8) Save setup and exit from programming

push button * to save

Note: one minute not operating any buttons, will automatically exit programming state.

how to use:

open by card: under normal working environment, door can be opened with 1 valid card.

open by card and password: door can be opened if correct password is input 10 seconds after card is read.

open by password: door can be opened by inputting user password.

Attention: the door controller supports 3 modes, opening by card, by password and by card and password.

management password initialize method:

without electricity, hold RESET button (not loosen), after electrify continued hold 3 seconds after ringing a buzzer loosen, at this time, the machine into user use sound state.

explanation: initialization operation is only modify system password for the default programming code on use of a password (12345, user password 8888, factory default), other data will not be deleted.

set authorization card and delete card methods:

without electricity, hold RESET button (not loosen), electrify RESET button don't loosen holding, brush card, the first piece of two for authorized card, the second for deleting card, then loosen ringing a buzzer. Right now, the machine into user use sound state.

explanation: the management card didn't set initialization manages passwords and open the door password.

Packing list:

access controller               1 set

User manual                  1 piece

Plastic insert for fixed and screw  1 bag

attachment 1: description for signal light

| issue type | red light | green light |
| --- | --- | --- |
| Normal working circumstance | glitter once per second |  |
| read valid card/correct password, unlock |  | light |
| open door and push button, then unlock |  | light |
| wrong password | light |  |
| invalid card | light |  |
| entry valid button | light |  |
| programming status | light | glitter quickly |

attachment2:description for buzzer sound

| issue type | buzzer |
| --- | --- |
| normal working status |  |
| read valid card/correct password, then unlock. | 2 short- toot |
| open door and push button, then unlock | 1 short-toot |
| wrong password | 3 short toots |
| invalid card | 3 short toots |
| entry valid button | 1 short-toot |
| door magnetic | continuous keep |
| programming status | 2 short- toots |
| entry correct programming code | 1 short toot |
| entry wrong programming code | 3 short toots |

Common question solutions

| common questions | may cause | solution |
| --- | --- | --- |
| reading card didn't sound | Card placed inductive zone for too long | leave sensor area read card again |
|  | cards have problems | change card to try |
| indicator not bright | Please check whether there is power | Change the power |
| Correct password or authorized card can't open the door | J1 jumpers set incorrectly | J1 jumpers set correctly |
|  | voltage is too low | adjusting voltage or power |
|  | electric current is too big, the power supply lock with can't afford it | for large current power |
|  | open the door model set wrong | open mode set correctly |
|  | lock problem | changing lock try |
| forget to open the door or management password |  | executive initialization operation |
| authorized has sent card |  | prior record card number, use card number increase card |
| the card can't normal authorized | card authorization or already repeated has reached its capacity |  |